| Project Name: | Linea |
| --- | --- |
| Document Title: | Design Rationale Summary |
| Document Number: | LIN-DRS-001 |
| Revision: | 0.0 |
| Revision Date: | 11/24/2025 |
| Prepared By: | GK |
| Supersedes: | N/A |
| Status: | Draft |
| Approval: | GK |
| Notes: | Document covers rationale through prototype RP3. Will be updated upon RP4 geometry stabilization. |

## Overview

This document summarizes the engineering and design rationale behind the Linea desk lamp, a premium adjustable lighting system developed for shared, multi-user office environments. The rationale outlines the functional requirements, architectural decisions, subsystem mechanisms, material selections, and user-interaction considerations that shaped the final product direction. The Linea lamp emphasizes durable construction, smooth and intuitive motion, and a refined visual language suited for a wide range of contemporary workspaces.

## Project Goals & Constraints

#### End-User Context

The original prompt required designing lighting for a collaborative office environment, where lamps are shared by multiple users throughout the day. The lamp must:

- accommodate a wide range of desk layouts

- blend into diverse visual environments (conservative, executive, creative)

- withstand long-term multi-user handling

- offer intuitive, low-cognitive-load controls

#### Experience Goals

- Provide function-forward lighting that “disappears” into the background of work

- Offer pleasant, adaptable user experience without overwhelming controls

- Deliver both focused task lighting and diffuse ambient illumination

- Create a sense of luxury and durability through material and CMF choices

- Maintain a conservative, minimal form that suits bulk office procurement

#### Branding Direction

The Linea brand emphasizes:

- Monochromatic anodized finishes

- Soft graphic treatments

- Debossed/enamel-filled details for tactile interaction

- Subtle premium cues rather than expressive ornamentation

#### Constraints

- The product must allow multiple base configurations (fixed mount, clamp, weighted base)

- The workspace coverage target is a 600 mm radius × 600 mm height cylinder

- Prototype budget limited high-fidelity metal parts; therefore early stages rely on mixed 3D-printed and machined components

- The lighting system should automatically adjust color temperature as a function of brightness, to avoid unnecessary control complexity

## System Architecture Rationale

#### Cartesian Workspace Approach

To maximize usability in shared environments, the lamp is built around a two-axis rail architecture:

- Vertical rail: adjusts height for seated or standing users

- Horizontal rail: adjusts reach and position across the desk

Combined, they provide Cartesian positioning of the light source over the workspace.

#### Rotational Base

A 360° rotational joint at the base introduces a third degree of freedom, enabling:

- one-handed repositioning

- coverage of the full cylindrical workspace

- easy adjustment from any chair orientation

#### Base Modularity

Linea was intentionally designed with swappable base types, mirroring configurable mic stands:

- Through-table mount

- Clamp mount

- Weighted base

This modularity supports workplace diversity without altering the core lamp architecture.

## Mechanism-Level Rationale

#### 4.1 Sliding Friction and Positioning Mechanism

Achieving smooth, silent motion with consistent positional stability was a primary objective for the Linea system. A belt-driven pulley coupled to a passive magnetic damping mechanism was selected to deliver this interaction quality. The moving pulley contains embedded neodymium magnets, while a stationary array within the housing provides a counter-field. The resulting magnetic attraction generates a controlled drag force that holds the lamp in place while still allowing effortless manual repositioning.

This solution offers several advantages over traditional mechanical detents or ratchet systems:

- Silent operation—no clicking or mechanical chatter

- Wear-free damping—no springs or pawls subject to fatigue

- Continuously variable resistance—unlike stepped ratchets

- Consistent long-term performance—magnetic forces do not degrade like friction pads

Although magnetic coupling is more complex than a tuned lobed ratchet, it provides a higher-end tactile experience appropriate for a premium shared-office lighting product. Additionally, the magnetic assembly can be partially showcased behind a windowed panel, reinforcing the product’s engineering identity without introducing noise or failure-prone complexity.

#### 4.2 Rail Motion System

The Linea lamp uses two orthogonal linear axes—vertical and horizontal—to provide a large workspace coverage area. Each axis employs precision stainless steel bearings riding directly on an anodized aluminum extrusion. Bearing preload is set using a hex-head eccentric cam, allowing fine adjustment of rolling resistance and eliminating backlash.

This architecture was selected because it provides:

- Smooth, low-noise operation suitable for shared environments

- Minimal maintenance requirements (sealed bearings, no lubrication paths)

- Tunable friction through preload control

- Consistent alignment over long spans

The decision to roll directly on aluminum—rather than adding steel rails—reduces weight, simplifies manufacturing, and maintains a clean, visually minimal form.

#### 4.3 Power Routing for High-Mobility Motion

Routing power through a moving two-axis system required a solution that avoided cable drag, twisting, or wire-bundle fatigue. To achieve this, Linea uses long, linear PCBs with exposed copper power traces, contacted by spring-loaded pogo pins integrated into the moving assemblies.

This approach enables:

- Unlimited travel along both rails without cable tangling

- Consistent electrical contact over the full range of motion

- A clean internal architecture free from cable chains or visible wiring

- Modular serviceability—PCBs can be replaced without disturbing mechanical assemblies

Although this solution carries a higher cost than traditional flex-cables, early testing demonstrated reliable contact under normal positioning, validating the architectural choice for a premium-grade product.

#### 4.4 Integrated Control Knob and Gear Interface

The brightness knob is visually integrated into the cylindrical column, maintaining the lamp’s monolithic silhouette. To achieve functional rotation without compromising the clean exterior, the knob features internal teeth that mesh with a compact gear mounted to the base-housed potentiometer.

This configuration provides:

- Seamless integration of controls into the form language

- Reliable torque transmission without wobble

- A stable path for wiring through the column interior

- Separation of the user interface from the electrical components, protecting electronics from user load

## Material & Finish Justification

#### Surface Look & Durability

A bead-blasted, anodized aluminum finish was selected for Linea’s external components because it is:

- premium

- scratch resistant

- fingerprint-resistant

- available in subtle monochromatic palettes

- consistent with modern office hardware

#### Branding Elements

Debossed features, enamel-filled marks, and restrained graphic treatments were used to create tactility and subtle premium cues.

## Electrical / Control Rationale

- Simple interface (1 knob) to reflect shared-office mentality

- Automatic color temperature adjustment to maintain proper visual comfort

- Internal components simplified for prototype stage; full PCB path available for production

## Manufacturing & Assembly Considerations

## Summary & Next Steps